EP 500 + SYLOMER® WALL MOUNT

Specification

Part 1 – General

  - Work Included

    - Supply of material, tools, and other equipment necessary to carry out the installation of the acoustic walls as shown on drawings.

  - System Description

- The acoustic walls shall be elastically suspended from the building structure, via acoustic wall mounts. The metal bracket of each wall mount shall be fixed to the building floor using the 2 fixation flanges of the mount, while the metal profile of the acoustic wall is fixed to the top metal plate of the wall mount. These wall mounts are conceived for withstanding vertical load of the drywall at a maximum load of 60Kg.

  - Quality

    - The manufacturing of the wall mounts shall be performed in a way that it fulfills its function as well as to comply with all applicable regulations.

  - Submittals

    - Product CAD drawings shall be provided to the designer for approval.

    - Product performance data shall be submitted to the designer for review and shall include an Airborne Sound Insulation Report according to ISO 10140-2 standard. The tests shall be performed in an independent laboratory. Test reports shall document a minimum Sound Transmission Class (STC) for a resiliently suspended wall.

Part 2 – Products

2.01 Materials

- The acoustic wall mounts shall be designed and manufactured by AMC MECANOCAUCHO®.

- The wall mount type and the load per mount shall be chosen to provide a natural frequency less than 20Hz.

Part 3 – Execution

3.01 Installation

- In places where rigid contact between the soundproofed wall and other non-elastically

suspended elements are present, elastic materials shall be used to avoid/eliminate the rigid connection that act as acoustic bridges.

- The installation of all sound isolation materials specified herein, shall be in accordance with procedures submitted by the isolation material manufacturer and approved by the architect.