VT TYPE D ACOUSTIC HANGER

Specification

Part 1 – General

  - Work Included

    - Supply of material, tools, and other equipment necessary to carry out the installation of acoustic ceilings as shown on drawings.

  - System Description

    - The acoustic ceilings shall be elastically suspended from the building structure, via acoustic hangers. The metal bracket of each hanger shall be fixed to the building structure using a screw or a stud bolt, while the acoustic ceiling hangs from the wire.

  - Quality

    - The manufacturing of the hangers shall be performed in a way that it fulfills its function as well as to comply with all applicable regulations.

  - Submittals

    - Product CAD drawings shall be provided to the designer for approval.

    - Product performance data shall be submitted to the designer for review and shall include an Airborne Sound Insulation Report according to ISO 10140-2 and an Impact Sound Level Report according to ISO 10140-3. The tests shall be performed in an independent laboratory. Test reports shall document a Sound Transmission Class (STC) and an Impact Isolation Class (IIC) for a resiliently suspended ceiling.

Part 2 – Products

2.01 Materials

- The acoustic hangers shall be designed and manufactured by AMC MECANOCAUCHO®.

- The acoustic hangers shall be able to continuously withstand the weight of the acoustic ceiling without suffering any significant settling after initial deflection.

- The acoustic hangers shall be manufactured following the fail-safe concept, even if the elastic element disappears completely, the acoustic hangers will keep its structural integrity.

- The bracket of the acoustic hanger shall be designed to allow for 10 degrees of vertical misalignment without making metal-to-metal contact between the hook and the bracket.

- The hanger type and the load per hanger shall be chosen to provide a natural frequency less than 10Hz.

Part 3 – Execution

3.01 Installation

- In places where rigid contact between the acoustic ceiling and other non-elastically suspended elements is present, elastic materials shall be used to avoid/eliminate the rigid connection that act as acoustic bridges.

- The installation of all sound isolation materials specified herein, shall be in accordance with procedures submitted by the isolation material manufacturer and approved by the architect.